Revision history of hardware promotion kit : HDK-EV730EA0_9V0

| Date | Revision | Modification | Modified by |
| --- | --- | --- | --- |
| 2022/05/18 | 1V0 | First released | Qizhi_tian |
| 2022/05/27 | 2V0 | Change decoupling capacitors from bottom layer to top layer | Qizhi_tian |
| 2022/06/14 | 3V0 | Update the network of UART_LOG_TX & UART_LOG_RX | Qizhi_tian |
| 2022/08/04 | 4V0 | Update bom (resisters of USER KEY, TVS), PL2303GC replace FT232 | Qizhi_tian |
| 2023/08/18 | 7V0 | Add PI circuits to the RFIO_A &RFIO_G, unit the PAD_G &PA_G of IC pad, Change the footprint of several capacitors from C0402 to C0201 of MIC circuits, such as C98, C99, C100, C101, C102, C103, C105, C107. NC R143 & R144 of auto-downloading circuit | Apple_song |
| 2023/11/17 | 7V2 | NC C155, change the value of DPX | Apple_song |
| 2024/05/29 | 8V0 | Correct the ESD named SD17 of Earphone Jack, Add a 0.1uF capacitor to CD pin of SD/EMMC. Change the silkscreen marker of DPX. Change the resistor value from 1MΩ to 47KΩ. add two 10pF capacitor but NC to the DP/DN, simultaneously close to Type-C connector. Change the value from 20K, 24K &51K to 24.9K, 34K & 47K of resistor and modify the circuit of ADC USER keys. New DPX added to the BOM. For BT & WLAN coexistence, relevant circuit are changed. Add a capacitor separately to PA13 & PA14 of GPIO connector. Change the relevant circuit of Audio earphone circuit. | Apple_song |
| 2024/07/15 | 8V1 | Change the part number of TVS of Audio from ESD5441 to Rclamp | Apple Song |
| 2025/05/19 | 8V2 | Update RFIO_G pi matching:C30=1pF,L14=3.3nH,C6=0.75pF; Update SYN Bypass cap:C51&C54=0.7pF Update SD16 silkscreen TOP Update Fabspec Excel | Gavin_Gao |
| 2025/07/30 | 9V0 | Modify LCD circuit, according to the need to add an external 7-inch screen | Gavin_Gao |